Supplementary Material:

In vitro metabolism of Δ8-, Δ9-, and Δ10-THC in human hepatocytes:  distinct Δ10-THC biotransformation and implications for drug testing

Robert Kronstranda,b, Henrik Greena,b, Markus Loha,c, Fabian Rüttimanna,c, Manuela Carla Montia,d

aDepartment of Biomedical and Clinical Science, Division of Clinical Chemistry and Pharmacology, Linköping University, Linköping, Sweden

bDepartment of Forensic Genetics and Forensic Toxicology, National Board of Forensic Medicine, Linköping, Sweden

cSchool of Life Sciences FHNW, University of Applied Sciences and Arts Northwestern Switzerland, Muttenz, Switzerland

dInstitute of Forensic Medicine, Department of Biomedical Engineering, University of Basel, Switzerland

*Corresponding author: manuela.monti@unibas.ch

Chapter 1: Mass spectrometer instrument settings 		p. 2

Chapter 2: MS2 spectra of ∆8-THC, ∆9-THC, and ∆10-THC		p. 3

Chapter 3: MS2 spectra of metabolites of ∆8-THC			p. 4-6

Chapter 4: MS2 spectra of metabolites of ∆9-THC			p. 7-8

Chapter 5: MS2 spectra of metabolites of ∆10-THC			p. 9

Chapter 6: OH-GLUC metabolites of ∆8-THC, ∆9-THC, and ∆10-THC	p. 10

Chapter 7: Example chromatograms of authentic urine sample		p. 11

Chapter 1: Mass spectrometer instrument settings

Table S1: Preferred Ion list.

| m/z [M+H]+ | Delta m/z (ppm) | Ret. Time (min) | Delta Ret. Time (min) | Collision Energy | Anticipated analyte | Formula |
| --- | --- | --- | --- | --- | --- | --- |
| 315.2319 | 20 | 6.21 | 0.5 | 20 | THC | C21H30O2 |
| 345.206 | 20 | 4.69 | 0.5 | 20 | THC-COOH | C21H28O4 |
| 331.2268 | 20 | 4.46 | 0.5 | 20 | 11-OH-THC | C21H30O3 |
| 521.2381 | 20 | 2.83 | 0.5 | 20 | THCCOOH gluc | C27H36O10 |
| 521.2381 | 20 | 3.08 | 0.5 | 20 | THCCOOH gluc | C27H36O10 |
| 507.2589 | 20 | 2.76 | 0.5 | 20 | OH-THC gluc | C27H38O9 |
| 491.2639 | 20 | 4.05 | 0.5 | 20 | THC gluc | C27H38O8 |
| 523.2538 | 20 | 3.21 | 0.5 | 20 | DiOH THC gluc | C27H38O10 |
| 347.2217 | 20 | 3.79 | 0.5 | 20 | DiOH | C21H30O4 |
| 347.2217 | 20 | 4.78 | 0.5 | 20 | DiOH | C21H30O4 |

Analytical method used for additional MS2 spectra

The hepatocyte samples were analyzed employing a complementary method to the one described in the main article. A 1290 Infinity ultra high-performance liquid chromatography (UHPLC) system coupled to a 6550 iFunnel Q quadrupole time-of-flight mass spectrometer (QToF-MS) by Agilent technologies (Sundbyberg, Sweden) was used. The HPLC system was equipped with the same column as described in the main article, namely, a Acquity HSS T3 column (150mm x 2.1mm, ID: 1.8µm) by Waters (Solna, Sweden).

Mobile phases consisted of 100% water with 0.5% formic acid (mobile phase A) and 100% methanol with 0.5% formic acid (mobile phase B). Chromatography was performed at 60 °C. An injection volume of 5 µL was used.

In Fig. S1 the gradient, mass acquisition, and source parameters are presented.

Fig. S1 Further details on the second analytical method.

Chapter 2: MS2 spectra of ∆8-THC, ∆9-THC, and ∆10-THC

Fig. S2: ∆8-THC

Fig. S3: ∆9-THC

Fig. S4: ∆10-THC

Chapter 3: MS2 spectra of metabolites of ∆8-THC

Fig. S5: ∆8-THC-OH

Fig. S6: ∆8-THC-OH-GLUC: a) zoomed in version also showing the ion at 299.1995 m/z, b) whole mass range

a)

b)

Fig. S7: ∆8-THC-COOH

Fig. S8: ∆8-THC-COOH-GLUC

Fig. S9: ∆8-THC-DiOH-GLUC

Note: The lower spectrum was obtained using the separate method described in supplementary material chapter 1.

Chapter 4: MS2 spectra of metabolites of ∆9-THC

Fig. S10: ∆9-THC-OH

Fig. S11: ∆9-THC-OH-GLUC 1

Note: Spectrum was obtained using the separate method described in supplementary material chapter 1.

Fig. S12: ∆9-THC-COOH

Fig. S13: ∆9-THC-COOH-GLUC

Chapter 5: MS2 spectra of metabolites of ∆10-THC

Fig. S14: ∆10-THC-GLUC

Fig. S15: ∆10-THC-OH-GLUC

Fig. S16: ∆10-THC-COOH

Chapter 6: OH-GLUC metabolites of ∆8-THC, ∆9-THC, and ∆10-THC

Fig. S17: Chromatograms showing the OH-GLUC metabolites for ∆8-THC, ∆9-THC, and ∆10-THC. Please note that the chromatograms are auto-scaled.

Chapter 7: Example chromatograms of authentic urine sample

Fig. S18: Chromatograms of an authentic urine sample.

∆9-THC (6.2 min)

∆8-THC-OH (4.3 min) and ∆9-THC-OH (4.5 min)

∆8-THC-COOH (4.6 min) and ∆9-THC-COOH (4.7 min)