SUPPLEMENTARY INFORMATION

Content: Methods

Supplementary Tables S1-S5

Quantification of endocannabinoids and lipid mediators in human plasma

Briefly, 500 µL of plasma were transferred to a 2 mL Precellys tube and were spiked with 20 µL of IS solution, containing deuterated endocannabinoids and lipidic mediators in a concentration of 1.25 µg/mL in acetonitrile. For the extraction of the molecules, 1000 µL of methyl tert-butyl ether (HPLC grade, Lab-Scan Analytical Science, Poland) were added to the tubes, and the mixture was homogenized in a Precellys 24 homogenizer coupled to a Cryolys (Bertin Technologies, France) at 4°C with 3 cycles of 20 seconds at 6500 rpm and 20 seconds of rest between cycles. Samples were centrifuged in an Allegra X-30R centrifugator (Beckam Coulter, USA) (1200 rpm, 5 minutes, 4°C). The organic layer was separated and evaporated to dryness under nitrogen (% 99.999, Messer, Germany) using a XcelVap evaporator (Horizon Technology, USA). Finally, the samples were reconstituted in 200 µL acetonitrile (ChromAR HPLC grade, Macron Fine Chemicals , USA)  and filtered with 0.22 µm polypropylene disks prior to HPLC-MS analysis.  Finally, 5 µL of the final acetonitrile solution were injected into the chromatographic system, and the content of the metabolites of interest was determined by external calibration.  The chromatographic set up included a 1290 Infinity II liquid chromatograph (Agilent Technologies, USA) coupled to a 6430 Triple Quad LC/MS (Agilent Technologies, USA) mass spectrometer. A Kinetex C18 precolumn 3 mm x 5 mm (Phenomenex, USA) and a Kinetex C18 column 15 x 3 mm, 2.6 µm (Phenomenex, USA) were used to provide the chromatographic separation of analytes.

For the preparation of IS solution and external calibration solutions the following standards were purchased from Cayman Chemicals (USA): 1-AG (95 %, 9:1 mixture of 1-AG and 2-AG), 2-AG (95 %, 9:1 mixture of 2-AG and 1-AG), NADA (98 %), LEA (98 %), OEA (98 %) , PEA (98 %), AEA (98 %), DEA (98 %), NADA-d8 (% 99), PEA-d4 (99%), 2-AG-d8 (99%), AEA-d8 (99%).

Chromatographic method and mass spectrometer conditions for the HPLC-MS analysis

The mobile phases used for the separation of analytes were A: Water (UHPLC-MS grade, Scharlab, Spain) with 0.1 % formic acid (Fischer Scientifics, USA) and B: MeOH (UHPLC-MS grade, Scharlab, Spain) with 0.1 % formic acid. The flow was 0.3 ml/min and the column was at 35ºC. The details of the chromatographic gradient are shown in the Table S4 at the Supplementary Material. The mass analysis was performed with an electrospray ionization source operating in positive mode (ESI+). N2 was used as drying gas, nebulization and collision gas, with a flow of 12 L/h, a temperature of 350 °C and a pressure of 30 psi; and the capillary voltage was of 2878 V. The precursor ion and product ions of the measured analytes was determined by individual analysis of each of the standards. At the same time, optimum fragmentor voltage, between 20 - 200 V; collision energy, between 5 – 45 eV; and cell accelerator voltage were determined. For this purpose, Optimizer software was employed (MassHunter, Agilent Technologies, USA) and precursor ion, product ions and ionization conditions of highest abundance were chosen as optimum. The chosen conditions for the analysis of each endocannabinoid are summarized in Table S5 at the Supplementary Material, together with the retention times of each of them.

Supplementary Table S1

Demographic characteristics of subjects with cannabis use disorder and matched controls.

Supplementary Table S2

Demographic characteristics and antipsychotic treatment of subjects with schizophrenia and matched controls.

Supplementary Table S3

Demographic characteristics and antipsychotic treatment of subjects with dual diagnostic and matched controls.

Supplementary Table S4

Solvent A composition change in the chromatographic method.

Supplementary Table S5

Retention times and chosen precursor ions, product ions, fragmentor voltages and collision energies for each of the measured analytes.