Supporting Information

Simulation of the metabolism of new psychoactive substances using electrochemistry-mass spectrometry: Introducing an innovative software tool for rapid data evaluation

Mark Wesnera, Steffen Heuckerotha, Michael Pützb and Uwe Karsta*

a Institute of Inorganic and Analytical Chemistry, University of Münster, Corrensstraße 48, 48149 Münster, Germany

b Federal Criminal Police Office, Forensic Science Institute, Äppelallee 45, 65203 Wiesbaden, Germany

* Corresponding author: uk@uni-muenster.de

# SI-1 Additional experimental details

Exact mass spectrometric parameters used for the detection of transformation products (TP) after metabolism simulation by electrochemistry coupled to high-resolution tandem mass spectrometry (EC-HRMS/MS) are listed in Table SI-1.1. Parameters used for data-dependent acquisition (DDA) of fragment spectra alongside the mass spectrometric detection are further listed in Table SI-1.2.

Table SI-1.1: Mass spectrometric parameters used for the detection of TPs after metabolism simulation by EC-HRMS/MS.

| Parameter | Description |
| --- | --- |
| Ionization mode | Positive ion electrospray |
| Spectra Rate | 4 Hz |
| Source |  |
| End Plate Offset | 500 V |
| Capillary | 3500 V |
| Nebulizer | 0.3 bar |
| Dry Gas | 4 l min-1 |
| Dry Temp | 200 °C |
| Tune |  |
| Deflection 1 Delta | 70 V |
| Funnel 1 RF | 250 Vpp |
| isCID energy | 0 eV |
| Funnel 2 RF | 500 Vpp |
| Multipole RF | 450 Vpp |
| Collision Energy | 5 eV |
| Collision RF | 550 Vpp |
| Ion Energy | 5 eV |
| Low Mass | 50 m/z |
| Transfer Time | 50 µs |
| Pre Puls Storage | 13 µs |

Table SI-1.2: Exact parameters used for DDA of fragment spectra of TPs generated by metabolism simulation utilizing EC-HRMS/MS.

| Parameter | Description |
| --- | --- |
| MS/MS Spectra |  |
| Rate Control | Same as MS spectrum |
| Multi CE | 50% & 100% |
| MS Filter |  |
| Normalized Threshold | 31 counts/1000 scans |
| Precursor Ion Selection |  |
| Selection by | Fixed number |
| Number of Precursors | 3 |
| Active Exclusion |  |
| Exclude after | 5 spectra |
| Release after | 1 min |
| Isolation & Fragmentation |  |
| Isolation Width | 1 m/z |
| CE | 30 eV |
| Fallback Charge State | 1 z |