Supplementary information: Characterization of new psychoactive substances by integrating benchtop NMR to multi-technique databases

Thomas Castaing-Cordiera, Alejandra Benavides Restrepoa, Damien Duboisb, Virginie Ladroueb, Fabrice Besacierc, Audrey Buleteb, Céline Charvozb, Anais Goupillea, Denis Jacquemina, Patrick Giraudeaua and Jonathan Farjona,*

aNantes Université, CNRS, CEISAM UMR 6230, F-44000 Nantes, France

bService National de police scientifique, Laboratoire de police scientifique de Lyon, 69134 Ecully, France

cService National de police scientifique, Sous-direction de la stratégie de l’innovation et du pilotage, 69134 Ecully, France

*Corresponding author: jonathan.farjon@univ-nantes.fr

S1: Database entries	4

S2: 1H-13C HSQC spectral comparison	16

S2: SNR comparison of different 1H-13C HSQC	17

S3: PH HSQC ME pulse program	18

S4: LF, HQI optimization	25

S5: 1D 1H 3-MMC peak search	27

S5: 1D 1H 4-MMC peak search	28

S5: 1D 1H 3-MMC absolute algorithm	29

S5: 1D 1H 3-MMC euclidian algorithm	30

S5: 1D 1H 4-MMC absolute algorithm	31

S5: 1D 1H 4-MMC euclidian algorithm	32

S5: 1D 19F AB-FUBINACA similarity search	33

S5: 1D 19F AB-FUBINACA peak search	34

S5: HSQC 3-MMC identification	35

S5: HSQC 4-MMC identification	36

S5: HSQC 4-MMC spectral homology	37

S5: IR 3-MMC peak search	38

S5: IR 3-MMC absolute algorithm	39

S5: IR 3-MMC euclidian algorithm	40

S5: IR 4-MMC peak search	41

S5: IR 4-MMC absolute algorithm	42

S5: IR 4-MMC euclidian algorithm	43

S6: 1D 19F substance n°1	44

S6: 2D HSQC substance n°1	45

S6: IR substance n°1	46

S6 : Identification substance 1 at 700MHz	47

S7: 2D HSQC substance n°2	50

S7: IR substance n°2	51

S7: Identification substance 2 at 700MHz	52

S8: 2D HSQC substance n°3	55

S8: 1D 1H substance n°3	56

S8: Identification substance 3 at 700MHz	57

S9: 2D HSQC identification substance n°4	58

S9: 2D HSQC elucidation substance n°4	59

S9: 1D 1H substance n°4	60

S9: IR substance n°4	61

S9: Elucidation substance n°4 at 700MHz	62

S10: 2D HSQC identification substance n°5	65

S10: 2D HSQC elucidation substance n°5	66

S10: 1D 1H substance n°5	67

S10: IR substance n°5	68

S10: HSQC prediction for MPHP	69

S10A: 700 MHz and predicted HSQC comparison	70

S10: Elucidation substance n°5 at 700MHz	71

S11: 2D HSQC identification substance n°6	74

S11: 2D HSQC elucidation substance n°6	75

S11: 1D 1H substance n°6	76

S11: Elucidation substance 6 at 700MHz	77

S12: Integrations area for the determination of purity	80

S1: Database entries

| Name | Structure | NMR concentration (mM) | NMR | IR |
| --- | --- | --- | --- | --- |
| 2-AI |  | 295 | Yes | Yes |
| 2-BMMP |  | 306 | Yes | Yes |
| 2C-E |  | 298 | Yes | Yes |
| 2C-I |  | 258 | Yes | Yes |
| 2-FDCK |  | 288 | Yes | Yes |
| 2C-B |  | x | No | Yes |
| 3-CMC |  | 291 | Yes | Yes |
| 3-FPM |  | 307 | Yes | Yes |
| 3-MMC |  | 299 | Yes | Yes |
| 4-CEC |  | 150 | Yes | Yes |
| 4-EPD |  | 315 | Yes | Yes |
| 4F-MPH |  | 317 | Yes | Yes |
| 4F-A |  | 273 | Yes | Yes |
| 4-MEC |  | 258 | Yes | Yes |
| 4-MeO-PCP |  | 305 | Yes | Yes |
| 4-MMC |  | 243 | Yes | Yes |
| 5-APB |  | 298 | Yes | Yes |
| 5F-ADB |  | 273 | Yes | Yes |
| 5-MeO-DALT |  | X | No | Yes |
| 5F-MDMB-PICA |  | 330 | Yes | Yes |
| 5F-PB-22 |  | 147 | Yes | Yes |
| AMT |  | 277 | Yes | Yes |
| α-PVP |  | 265 | Yes | Yes |
| AB-CHMINACA |  | 312 | Yes | Yes |
| AB-FUBINACA |  | 291 | Yes | Yes |
| ADB-CHMINACA |  | 279 | Yes | Yes |
| AF |  | 276 | Yes | Yes |
| AMB-CHMICA |  | 275 | Yes | Yes |
| AMB-FUBINACA |  | 248 | Yes | Yes |
| AMF |  | 236 | Yes | Yes |
| AMP |  | 300 | Yes | Yes |
| APINACA |  | 301 | Yes | Yes |
| bk-MBDB |  | X | No | Yes |
| bk-MPA |  | 286 | Yes | Yes |
| BPR |  | 319 | Yes | Yes |
| DMPEA-NBTOMe |  | 268 | Yes | Yes |
| DPD |  | 297 | Yes | Yes |
| ETH-CAT |  | 310 | Yes | Yes |
| FIBF |  | 206 | Yes | Yes |
| HEXEN |  | 310 | Yes | Yes |
| JWH210 |  | 265 | Yes | Yes |
| MDAI |  | 275 | Yes | Yes |
| MDEC |  | 295 | Yes | Yes |
| MDMB-FUBINACA |  | 290 | Yes | Yes |
| MDMB-CHMCZCA |  | 300 | Yes | Yes |
| MDMB-CHMICA |  | 260 | Yes | Yes |
| MDMC |  | 310 | Yes | Yes |
| MDPV |  | 110 | Yes | Yes |
| MPHP |  | 200 | Yes | Yes |
| MXE |  | 144 | Yes | Yes |
| NEK |  | 203 | Yes | Yes |
| O-PCE |  | 150 | Yes | Yes |
| PHENF |  | 217 | Yes | Yes |
| PV9 |  | 250 | Yes | Yes |
| TFMPP |  | 295 | Yes | Yes |
| U-47700 |  | 300 | Yes | Yes |
| UR-144 |  | 306 | Yes | Yes |

S2: 1H-13C HSQC spectral comparison

S2: SNR comparison of different 1H-13C HSQC

S3: PH HSQC ME pulse program

Procedure (pulse_program,dir,mode)

# Interface description (name, label, x, y, ctrl, vartype)

interface = ["nucleus", "Nuclei","0","0", "tb", "readonly_string",

"b1Freq1H", "Proton Frequency (MHz)","0","0", "tbw","freq",

"offFreq", "Offset Frequency (Hz)","0","1", "tbw","float",

"pulseLength13C", "Pulse length for all(us)", "0","2", "tbw","pulselength",

"90AmplitudeHC", "90 Pulse amplitude (dB)",  "0","3", "tbw","pulseamp",

"180AmplitudeHC", "180 Pulse amplitude (dB)", "0","4", "tbw","pulseamp",

"b1Freq13C", "Carbon Frequency (MHz)",   "1","0", "tbw","freq",

"90Amplitude13C", "90 Pulse amplitude (dB)",  "1","1", "tbw","pulseamp",

"spoilAmp", "Homospoil amplitude", "1","2", "tbw","float,[1,1e4]",

"spoilDur", "Homospoil duration (us)",  "1","3", "tbw","sdelay",

"gradStab", "Grad ringdown time (us)",  "1","4", "tb", "sdelay",

"aCDec", "Decouple amplitude (dB)",  "1","5", "tbw","pulseamp",

"pulseLengthC180", "Decouple length 180 (us)", "1","6", "tbw","pulselength",

"jch", "J(C,H) (Hz)", "2","2", "tbw","float",

"bandwidth2", "Bandwidth2 (kHz)", "2","3", "tbw","float,[1,1e5]",

"nrSteps","Number of steps","2","4", "tbw","integer,[1,1024]",

"repTime","Repetition time (ms)", "3","1", "tbw","reptime",

"dummyCycles", "Number of dummy cycles",  "3","2", "tb", "integer,[0,16]",

"gradStabEnd", "Grad ringdown time (us)",  "4","4", "tb", "sdelay"

# Relationships to determine remaining variable values

relationships = ["b1Freq = b1Freq1H",

"d9 = 1",

"d4 = 1",

"n1 = nrPnts",

"n3 = nrEchoes",

"n4 = xshim",

"n5 = yshim",

"n6 = zshim",

"n7 = xshim-(spoilAmp*xshim/(abs(xshim)+0.1))",

"n8 = yshim-(spoilAmp*yshim/(abs(yshim)+0.1))",

"n9 = zshim-(spoilAmp*zshim/(abs(zshim)+0.1))",

"n10 = xshim-(0.75*spoilAmp*xshim/(abs(xshim)+0.1))",

"n11 = yshim-(0.0*spoilAmp*yshim/(abs(yshim)+0.1))",

"n12 = zshim-(0.0*spoilAmp*zshim/(abs(zshim)+0.1))",

"n13 = xshim+(spoilAmp*xshim/(abs(xshim)+0.1))",

"n14 = yshim+(spoilAmp*yshim/(abs(yshim)+0.1))",

"n15 = zshim+(spoilAmp*zshim/(abs(zshim)+0.1))",

"a1 = 90AmplitudeHC",

"a2 = 180AmplitudeHC",

"a3 = 90Amplitude13C",

"a4 = 0",

"a5 = -85",

"a6 = aCDec",

"d1 = pulseLength13C",

"d12 = pulseLengthC180/2",

"d13 = 3*pulseLengthC180/2",

"d14 = 2*pulseLengthC180",

"d6 = pulseLengthC180",

"d7 = spoilDur",

"d8 = gradStab",         # gradient settle delay

"d2 = 1e6/4/jch-2*pulseLength13C/3.14",

"d3 = 1e6/3/jch-pgo-pulseLength13C/2",

"d10 = 1e6/2/jch-pgo-2*pulseLength13C/3.14",

"d20 = 3.976/4*d7",

"w1 = 1e3/2/bandwidth2",

"d5 = acqDelay",

"O1 = offFreq",

"f1 = double(b1Freq1H)+double(offFreq/1e6d)",

"f2 = 10d*f1",

"f3 = double(b1Freq13C)",

"a111 = -80",

"totPnts = nrPnts",

"totTime = acqTime"]

# Define the tabs and their order

tabs = ["Pulse_sequence","Progress","Acquisition2d",

"Processing_Std","Display_2D","File_Settings"]

# These parameters will be changed between experiments

variables = ["w1"]

# x and y spacing between controls

dim = [190,26]

# Pulse sequence

initpp(dir) # Reset internal parameter list

cleardata(n1)

settxfreqs(f1,f3)

delay(5000)                   # allow time to finish lock scan

#BIRD module

pulse(1,a1,p1,d1)  # 90 H RF pulse, phase 0

delay(d10)                     # 1/2J

pulse(1,a1,p3,f1,2,a3,p3,f3,d1)  # 90 HC pulse

delay(1)                     # rf delay

pulse(1,a2,p1,f1,2,a4,p1,f3,d1)  # 180 HC pulse

delay(1)                     # rf delay

pulse(1,a1,p3,f1,2,a3,p3,f3,d1)  # 90 HC pulse

delay(d10)                     # 1/2J

pulse(1,a1,p13,d1)  # 90 H RF pulse, phase 2

#350 ms delay hardcoded for now

delay(175000)       #nulling delay

delay(175000)       #nulling delay

pulse(1,a1,p1,d1)  # 90 H RF pulse

delay(d2)                     # coupling delay

pulse(1,a1,p2,f1,2,a3,p5,f3,d1)  # 90 HC pulse

delay(1)                     # rf delay

pulse(1,a2,p3,f1,2,a4,p7,f3,d1)  # 180 HC pulse

delay(1)                     # rf delay

pulse(1,a1,p2,f1,2,a3,p5,f3,d1)  # 90 HC pulse

delay(d2)                     # coupling delay

#Split HC pulse into separate H and C pulses - Z filter between two

pulse(1,a1,p3,d1)  # 90 H RF pulse

delay(10)

shim16(1,n7)    # x

shim16(2,n8)    # y

shim16(0,n9)    # z

delay(d7)                     # homospoil delay

delay(d7)                     # homospoil delay

delay(d7)                     # homospoil delay

delay(d7)                     # homospoil delay

delay(d7)                     # homospoil delay

shim16(1,n4)    # x

shim16(2,n5)    # y

shim16(0,n6)    # z

delay(d8)                     # settle delay

pulse(2,a3,p8,d1)  # 90 C RF pulse

wait(w1)                      # evolution delay

pulse(1,a1,p5,d1)  # 90 H pulse

delay(1)                     # rf delay

pulse(1,a2,p4,d1)  # 180 H pulse

delay(1)                     # rf delay

pulse(1,a1,p5,d1)  # 90 H pulse

wait(w1)                      # evolution delay

#Mutiplicity-editing

delay(d10)                     # 1/2J

pulse(1,a1,p3,f1,2,a3,p3,f3,d1)  # 90 HC pulse

delay(1)                     # rf delay

pulse(1,a2,p1,f1,2,a4,p1,f3,d1)  # 180 HC pulse

delay(1)                     # rf delay

pulse(1,a1,p3,f1,2,a3,p3,f3,d1)  # 90 HC pulse

delay(d10)                     # 1/2J

#End of ME

#Split HC pulses and use Z filter again

pulse(2,a3,p9,d1)  # 90 C RF pulse

delay(10)

shim16(1,n7)    # x

shim16(2,n8)    # y

shim16(0,n9)    # z

delay(d7)                     # homospoil delay

delay(d7)                     # homospoil delay

delay(d7)                     # homospoil delay

delay(d7)                     # homospoil delay

shim16(1,n4)    # x

shim16(2,n5)    # y

shim16(0,n6)    # z

delay(d8)                     # settle delay

pulse(1,a1,p5,d1)  # 90 H RF pulse

delay(d2)                     # coupling delay

pulse(1,a1,p3,f1,2,a3,p5,f3,d1)  # 90 HC pulse

delay(1)                     # rf delay

pulse(1,a2,p6,f1,2,a4,p10,f3,d1)  # 180 HC pulse

delay(1)                     # rf delay

pulse(1,a1,p3,f1,2,a3,p5,f3,d1)  # 90 HC pulse

delay(d2)                     # coupling delay

#Acquisition and decoupling

acquireon(n1)

loop(l1,n2)

pulse(2,a6,p12,d13); delay(d9); pulse(2,a6,p11,d14); delay(d9); pulse(2,a6,p12,d6); delay(d9); pulse(2,a6,p11,d13); delay(d9); pulse(2,a6,p12,d12); delay(d9); pulse(2,a6,p11,d6); delay(d9); pulse(2,a6,p12,d14); delay(d9); pulse(2,a6,p11,d6); delay(d9); pulse(2,a6,p12,d13);

delay(d4);

pulse(2,a6,p11,d13); delay(d9); pulse(2,a6,p12,d14); delay(d9); pulse(2,a6,p11,d6); delay(d9); pulse(2,a6,p12,d13); delay(d9); pulse(2,a6,p11,d12); delay(d9); pulse(2,a6,p12,d6); delay(d9); pulse(2,a6,p11,d14); delay(d9); pulse(2,a6,p12,d6); delay(d9); pulse(2,a6,p11,d13);

delay(d4);

pulse(2,a6,p11,d13); delay(d9); pulse(2,a6,p12,d14); delay(d9); pulse(2,a6,p11,d6); delay(d9); pulse(2,a6,p12,d13); delay(d9); pulse(2,a6,p11,d12); delay(d9); pulse(2,a6,p12,d6); delay(d9); pulse(2,a6,p11,d14); delay(d9); pulse(2,a6,p12,d6); delay(d9); pulse(2,a6,p11,d13);

delay(d4);

pulse(2,a6,p12,d13); delay(d9); pulse(2,a6,p11,d14); delay(d9); pulse(2,a6,p12,d6); delay(d9); pulse(2,a6,p11,d13); delay(d9); pulse(2,a6,p12,d12); delay(d9); pulse(2,a6,p11,d6); delay(d9); pulse(2,a6,p12,d14); delay(d9); pulse(2,a6,p11,d6); delay(d9); pulse(2,a6,p12,d13);

delay(d4);

delay(10);

endloop(l1)

acquireoff("overwrite",n1)

shim16(1,n7)    # x

shim16(2,n8)    # y

shim16(0,n9)    # z

delay(100000)                     # homospoil delay

shim16(1,n4)    # x

shim16(2,n5)    # y

shim16(0,n6)    # z

delay(50000)

lst = endpp(mode) # Return parameter list

# Phase cycle list

phaseList  = [0,0,0,0;   # phase 1

0,0,0,0;   # phase 2

1,1,1,1;   # phase 3

0,0,0,0;   # phase 4

1,1,1,1;   # phase 5

0,0,0,0;   # phase 6

0,0,0,0;   # phase 7

0,2,0,2;   # phase 8

0,0,2,2;   # phase 9

0,0,0,0;   # phase 10

0,0,0,0;   # Decouple phase 1

2,2,2,2;   # Decouple phase 2

2,2,2,2;   # BIRD

0,2,2,0]   # Acquire phase

endproc(lst,tabs,interface,relationships,variables,dim,phaseList)

S4: LF, HQI optimization

For 1D 1H, we evaluated both types of search comparison on 3-MMC and 4-MMC but the best results were obtained with the similarity search applied to a spectral width between -1 and 12 ppm. This is due to the fact that 60 MHz 1H NMR spectra are overcrowded and highly hampered by overlapping, which makes the peak searching method inefficient. In most cases, a large part of the database is returned by the software with an HQI between 0 and 100, therefore only the first four results will be presented. For the identification of 3-MMC, the following results were obtained with an absolute similarity search (HQI values are indicated in brackets for each hit): 3-MMC (96.45), 4-MMC (94.94), 3-CMC (91.79) and 4-MEC (90.13). The following results were obtained with a Euclidian search: 4-MMC (74.81), 3-CMC (69.99), 4-MEC (67.85) and 3-MMC (66.76). While the first four results were from the cathinone family in both cases, only the absolute search provided the correct molecule as a first hit. For 4-MMC, the absolute similarity search provided as match 4-MMC (100), 3-MMC (95.41), bk-MPA (91.23), 4-MEC (90.64) while the Euclidian similarity search returned 4-MMC (81.89), 4-MEC (68.02), 3-MMC (65.98), 3-CMC (65.73). While the first four results were from the cathinone family again, only the absolute search was able to provide 4-MMC with an HQI of 100 and 3-MMC as a second match. Therefore, absolute search was chosen as a default approach for all 1D 1H spectra. With this pre-validation, 1D 1H NMR can be used to identify but also to find substances close to the unknown one.

For AB-FUBINACA, the 1D 19F spectrum was compared with similarity and peak search. For similarity search with absolute algorithm, the first four matches were AB-FUBINACA (92.21), MDMB-FUBINACA (92.20), AMB-FUBINACA (91.87) and 3-FPM (91.46), while for the peak search only 3 molecules were returned with a LF fixed at 0.3 ppm: MDMB-FUBINACA (100), AMB-FUBINACA (100) and AB-FUBINACA (100). Therefore, peak search was found more efficient, probably because it also considers the multiplicity of the NMR signal.

For HSQC comparison, peak search was chosen by default because it is the only approach available in the software to compare HSQC maps. A first LF value needs to be set for NPS identification. In the 3-MMC / 4-MMC example, a single match should ideally be obtained. Such results were obtained with a LF of 0.1 ppm for 1H and 1 ppm for 13C. The next LF that must be chosen is the one for spectral homology comparison. This one must allow to obtain at least one another match for the 4-MMC. 3-MMC was found with an HQI of 100 if LF of 0.2 for 1H and 2 ppm for 13C.

Finally, for IR spectroscopy, comparison by peak for 3-MMC was tested but gave disappointing results. In fact, HQI was not calculated and the 4 first results were 3-MMC, APINACA, ADB-CHMINACA, 4-MMC. The second and the last matches were from cannabinoids family, so this type of comparison was excluded. Then absolute and Euclidian algorithm with similarity search were compared for the two pre validated molecules. For absolute classification of 3-MMC, the first matches were 3-MMC (87.36), 4-MMC (62.23), 4-MEC (60.02) and bk-MPA (52.91) while with Euclidian, 3-MMC (88.98), 4-MMC (60.54), 4-MEC (56.70) and bk-MPA (55.35) were obtained. For identification of 3-MMC, the largest difference between the two isomers was obtained with the Euclidian algorithm. The IR comparison of 4-MMC with absolute algorithm gave 4-MMC (100.00), 4-MEC (70.79), 4-EPD (62.56) and bk-MPA (60.76) as match while the Euclidian algorithm returned 4-MMC (100.00), 4-MEC (67.57) a homologous lateral chain of 4-MMC, 3-MMC (57.94) a positional isomer and 5F-ADB (57.10).To find NPS close to the unknown one, it seems that the Euclidian search is more correct since the found hits are very close to 4-MMC most of them have a substituent in para of their aromatic ring.

The pre validation step allowed to find the best parameters for comparison. Thus, for the next steps the database comparison was performed with similarity search with Absolute algorithm for 1D 1H spectra, peak search for 1D 19F spectra, and similarity search with Euclidian algorithm for IR spectroscopy. In the case of HSQC, peak search with LF 0.1/1 ppm was chosen for identification and at least 0.2/2 ppm for elucidation.

S5: 1D 1H 3-MMC peak search

S5: 1D 1H 4-MMC peak search

S5: 1D 1H 3-MMC absolute algorithm

S5: 1D 1H 3-MMC euclidian algorithm

S5: 1D 1H 4-MMC absolute algorithm

S5: 1D 1H 4-MMC euclidian algorithm

S5: 1D 19F AB-FUBINACA similarity search

S5: 1D 19F AB-FUBINACA peak search

S5: HSQC 3-MMC identification

S5: HSQC 4-MMC identification

S5: HSQC 4-MMC spectral homology

S5: IR 3-MMC peak search

S5: IR 3-MMC absolute algorithm

S5: IR 3-MMC euclidian algorithm

S5: IR 4-MMC peak search

S5: IR 4-MMC absolute algorithm

S5: IR 4-MMC euclidian algorithm

S6: 1D 19F substance n°1

S6: 2D HSQC substance n°1

S6: IR substance n°1

S6 : Identification substance 1 at 700MHz

1D 1H

1H-1H COSY

1H-13C HSQC

S7: 2D HSQC substance n°2

S7: IR substance n°2

S7: Identification substance 2 at 700MHz

1D 1H

1H-1H COSY

1H-13C HSQC

S8: 2D HSQC substance n°3

S8: 1D 1H substance n°3

S8: Identification substance 3 at 700MHz

1D 1H NMR comparison with pure samples

S9: 2D HSQC identification substance n°4

S9: 2D HSQC elucidation substance n°4

S9: 1D 1H substance n°4

S9: IR substance n°4

S9: Elucidation substance n°4 at 700MHz

1D 1H

1H-1H COSY

1H-13C HSQC

S10: 2D HSQC identification substance n°5

S10: 2D HSQC elucidation substance n°5

S10: 1D 1H substance n°5

S10: IR substance n°5

S10: HSQC prediction for MPHP

S10A: 700 MHz and predicted HSQC comparison

S10: Elucidation substance n°5 at 700MHz

1D 1H

1H-1H COSY

1H-13C HSQC

S11: 2D HSQC identification substance n°6

S11: 2D HSQC elucidation substance n°6

S11: 1D 1H substance n°6

S11: Elucidation substance 6 at 700MHz

1D 1H

1H-1H COSY

1H-13C HSQC

S12: Integrations area for the determination of purity