Supplementary Information

Rapid LC-QTOF-MS screening method for semi-synthetic cannabinoids in whole blood

Table of contents

- Table Recovery rates and matrix effects SPE, and LLE in glass vials	S1

- Recovery rates from the LLE protocol, measured in PP vials	S2

- Recovery rates from the LLE protocol, measured in glass vials	S3

- Recovery rates from the SPE protocol, measured in PP vials	S4

- Δ9-THC-COOH interference in a real case sample (Δ8-THC-COOH)	S5

- Δ9-THC and Δ8-THC in a real case sample	S6

- Δ9-THC and Δ8-THC in a real case sample	S7

- HHC in a real case sample	S8

S1: Recovery rates and matrix effects of the analytes from the liquid-liquid extraction protocol (LLE), measured in glass vials (left side) and from the solid-phase extraction protocol (SPE), measured in polypropylene vials (right side). Graphical illustration of the recovery rates from the LLE protocol (analyzed in glass vials) are shown in S3. Graphical illustration of the recovery rates from the SPE protocol are shown in S4.

| Analyte | Recovery rates LLE glass vials / % | Matrix effects LLE glass vials / % | Recovery rates SPE / % | Matrix effects SPE / % |
| --- | --- | --- | --- | --- |
| Δ9-THCO | 79 ± 7.3 | 76 | 69 ± 6.2 | 97 |
| 9β-OH-HHC | 55 ± 4.8 | 49 | 82 ± 2.4 | 54 |
| 11-OH-Δ9-THC | 96 ± 10.5 | 39 | 75 ± 2.0 | 76 |
| 11-OH-Δ8-THC | 61 ± 7.7 | 45 | 86 ± 12.1 | 46 |
| Δ9-THCE | 91 ± 1.3 | 36 | 75 ± 7.3 | 85 |
| 11-COOH-9R-HHC | 62 ± 6.4 | 27 | 90 ± 1.1 | 33 |
| 11-COOH-Δ9-THC | 99 ± 8.3 | 28 | 64 ± 4.4 | 117 |
| 9α-OH-HHC | 96 ±8.7 | 48 | 66 ± 4.4 | 95 |
| CBD | 65 ± 10.8 | 18 | 81 ± 5.8 | 79 |
| CBG | 94 ± 16.3 | 15 | 62 ± 6.8 | 126 |
| Δ9-THCB | 103 ± 15.3 | 22 | 63 ± 3.6 | 96 |
| CBN | 111 ± 26.5 | 14 | 63 ± 2.2 | 84 |
| Δ9-THC | 74 ± 16.0 | 11 | 68 ± 5.7 | 39 |
| Δ8-THC | 95 ± 31.8 | 15 | 61 ± 2.4 | 43 |
| 9S-HHC | 113 ± 30.5 | 12 | 53 ± 2.3 | 73 |
| 9R-HHC | 0 | N/A | 69 ± 2.7 | 54 |
| Δ9-THCH | 101 ± 29.9 | 31 | 72 ± 3.4 | 171 |
| 9R-HHCH | 0 | N/A | 86 ± 1.8 | 85 |
| Δ9-THCP | 0 | N/A | 45 ± 4.6 | 52 |
| Δ9-THC-O | 0 | N/A | 57 ± 2.5 | 148 |
| 9R-HHCP | 0 | N/A | 44 ± 3.0 | 52 |
| 9R-HHC-O | 0 | N/A | 58 ± 1.7 | 74 |
| Δ9-THC-C8 | 0 | N/A | 31 ± 4.0 | 97 |
| Δ9-THC-OMe | 0 | N/A | 28 ± 2.8 | 120 |

S2: Recovery rates of the analytes included in the screening panel. Analytes were spiked on blank blood and were extracted with a liquid-liquid-extraction (LLE) protocol. The solutions were measured using polypropylene (PP) vials. Recovery rates and error bars were determined from triplicates.

S3: Recovery rates of the analytes included in the screening panel. Analytes were spiked on blank blood and were extracted with a liquid-liquid-extraction protocol. The solutions were measured using glass vials. The recovery rates were in general lower than from PP vials and the late eluting compounds were not detected. Recovery rates and error bars were determined from triplicates.

S4: Recovery rates of the analytes included in the screening panel. Analytes were spiked on blank blood and were extracted with a solid-phase-extraction (SPE) protocol. The solutions were measured using PP vials. Recovery rates seem to drop with the lipophilicity of the analytes. Recovery rates and error bars were determined from triplicates.

S5: Real case sample, which showed an interference for Δ9-THC-COOH at Rt = 9.99 min, presumably Δ8-THC-COOH. The peaks are not fully resolved.

S6: A real case sample with similar amounts of Δ8-THC and Δ9-THC. The peaks are not fully resolved.

S7: Another real case sample showing more Δ8-THC than Δ9-THC. The isomers are not fully resolved.

S8: A real case sample containing HHC. Differentiation between the epimers (9R)- and (9S)-HHC is not possible as they are coeluting.